Smart Renewable Energy Monitoring and Grid Optimization Platform

A REPORT

In

CSA1012 –Software Engineering

Submitted in the partial fulfillment for the award of the degree

Of

BACHELOR OF ENGINEERING

IN

INFORMATION TECHNOLOGY

Submitted by:

N. Srikanth (192411139)

Supervisor/Advisor Name:
Dr. K. Anita Devamani,

Professor/CSE

SAVEETHA INSTITUTE OF MEDICAL AND TECHNICAL SCIENCES

CHENNAI-602105

CO4 – AT1
SOFTWARE TESTING – TEST CASE DESIGN

Smart Renewable Energy Monitoring and Grid Optimization Platform

# 1. Introduction

## 1.1 Background

The increasing adoption of renewable energy sources such as solar and wind power has created a need for intelligent systems that can continuously monitor energy generation, consumption, storage, and grid conditions. Traditional energy monitoring systems mainly display generation and consumption information but provide limited support for prediction, fault detection, and intelligent energy distribution.

The proposed Smart Renewable Energy Monitoring and Grid Optimization Platform provides a centralized platform for monitoring renewable energy generation and electricity consumption in real time. The system collects data from renewable-energy sensors, stores and analyzes the data, predicts future energy generation, compares energy supply with demand, and supports optimized grid-energy management.

## 1.2 Smart Renewable Energy Platform Overview

The platform monitors renewable energy sources, collects sensor readings, analyzes energy generation and consumption, predicts future renewable generation, and optimizes energy distribution.

Renewable Sources → Sensor Data Collection → Data Validation → Database → Energy Analysis → Renewable Energy Prediction → Grid Optimization → Dashboard + Alerts + Reports

## 1.3 Purpose of Testing

Because the platform handles real-time energy information and optimization decisions, incorrect data or delayed processing may result in incorrect energy-management decisions. The system must correctly validate sensor readings, calculate energy values, generate predictions, identify supply-demand conditions, and produce appropriate alerts. Systematic testing is essential to verify normal, invalid, boundary, exceptional, concurrent, and failure conditions.

## 1.4 Testing Objectives

Verify functional correctness of all core platform modules.

Verify correct collection and validation of renewable-energy sensor data.

Verify accurate monitoring of energy generation and consumption.

Verify correctness of renewable-energy prediction.

Verify correct comparison between energy generation and demand.

Verify correct grid optimization decisions.

Verify appropriate warning and critical alerts.

Verify correct handling of sensor, network, database, and API failures.

Verify security and authorization of the platform.

Ensure traceability between requirements, test scenarios, and test cases.

## 1.5 Scope of Testing

User authentication and session management

Renewable-energy sensor monitoring

Energy generation and consumption monitoring

Sensor data validation and storage

Renewable-energy prediction

Supply-demand analysis and grid optimization

Energy alerts and notifications

Dashboard and visualization

Energy reports

Security, reliability, data integrity, performance, scalability, usability, and compatibility

# 2. System Overview and Architecture

The platform is organized into modules that transform raw renewable-energy sensor readings into monitoring, prediction, and grid-management decisions. Sensors collect generation and consumption information. The data is transmitted, validated, and stored. Analysis and prediction modules process the data, and the optimization module compares generation with demand and determines the appropriate energy-distribution strategy.

Smart Renewable Energy Monitoring and Grid Optimization Platform Architecture

SOLAR / WIND SOURCES
↓
IoT SENSORS
↓
DATA COLLECTION
↓
DATA VALIDATION → DATABASE STORAGE
↓
ENERGY ANALYSIS
↓
RENEWABLE ENERGY PREDICTION
↓
SUPPLY–DEMAND ANALYSIS
↓
GRID OPTIMIZATION
↓
DASHBOARD + ALERTS + REPORTS

Energy Data Processing Workflow

Sensor Generates Reading → Validate Sensor Data → Store Energy Data → Calculate Generation & Consumption → Predict Renewable Energy → Compare Generation with Demand → Optimize Energy Distribution → Update Dashboard + Generate Alerts

# 3. Functional Requirements

| Req. ID | Functional Requirement | Module | Priority |
| --- | --- | --- | --- |
| FR-01 | User Registration and Login | Authentication | High |
| FR-02 | Password Reset | Authentication | Medium |
| FR-03 | Renewable Sensor Registration | Sensor Management | High |
| FR-04 | Sensor Data Collection | Monitoring | High |
| FR-05 | Energy Generation Monitoring | Monitoring | High |
| FR-06 | Energy Consumption Monitoring | Monitoring | High |
| FR-07 | Sensor Data Validation | Data Management | High |
| FR-08 | Energy Data Storage | Data Management | High |
| FR-09 | Renewable Energy Prediction | Prediction | High |
| FR-10 | Generation-Demand Comparison | Energy Analysis | High |
| FR-11 | Grid Optimization | Grid Management | High |
| FR-12 | Real-Time Dashboard Updates | Visualization | High |
| FR-13 | Abnormal Energy Detection | Monitoring | Medium |
| FR-14 | Warning and Critical Alerts | Notification | High |
| FR-15 | Sensor Failure Detection | Monitoring | Medium |
| FR-16 | Energy Report Generation | Reporting | Medium |
| FR-17 | Energy History Visualization | Visualization | Medium |
| FR-18 | Admin Monitoring Panel | Administration | Medium |
| FR-19 | Energy Usage Analytics | Analytics | Medium |
| FR-20 | Grid Status Monitoring | Grid Management | High |
| FR-21 | Data Export | Reporting | Medium |

# 4. Non-Functional Requirements

| ID | Requirement | Description | Testing Objective |
| --- | --- | --- | --- |
| NFR-01 | Performance | Process and display energy readings within an acceptable response time. | Verify response time and dashboard update speed. |
| NFR-02 | Security | Restrict access to authorized users and protect energy data. | Verify authentication and authorization. |
| NFR-03 | Reliability | Continue functioning correctly during sensor, network, API, or database failures. | Verify failure handling and recovery. |
| NFR-04 | Availability | Remain available for continuous energy monitoring. | Verify accessibility and recovery. |
| NFR-05 | Usability | Present energy information, charts, alerts, and status clearly. | Verify interface clarity and ease of use. |
| NFR-06 | Scalability | Support increasing sensors and energy data without degradation. | Verify behaviour under increasing load. |
| NFR-07 | Data Integrity | Maintain accurate readings without unintended duplication or loss. | Verify consistency and correctness. |
| NFR-08 | Maintainability | Allow modules to be independently maintained and tested. | Verify modular behaviour. |
| NFR-09 | Compatibility | Work correctly across supported browsers and devices. | Verify cross-platform behaviour. |
| NFR-10 | Privacy | Prevent exposure of user and energy information to unauthorized users. | Verify data isolation and access control. |

# 5. Test Scenario Identification

| Scenario ID | Test Scenario |
| --- | --- |
| TS-01 | Valid user login |
| TS-02 | Invalid user login |
| TS-03 | Sensor registration |
| TS-04 | Renewable-energy data collection |
| TS-05 | Energy generation monitoring |
| TS-06 | Energy consumption monitoring |
| TS-07 | Valid sensor data validation |
| TS-08 | Invalid sensor data validation |
| TS-09 | Energy data storage |
| TS-10 | Renewable-energy prediction |
| TS-11 | Generation-demand comparison |
| TS-12 | Grid optimization |
| TS-13 | Abnormal energy detection |
| TS-14 | Warning and critical alerts |
| TS-15 | Sensor failure handling |
| TS-16 | Dashboard visualization |
| TS-17 | Report generation |
| TS-18 | Grid status monitoring |
| TS-19 | Security and unauthorized access |
| TS-20 | Network/API/database failure |
| TS-21 | Concurrent sensor data |
| TS-22 | Data duplication handling |

# 6. Test Design Techniques

Four complementary test-design techniques are applied: Equivalence Partitioning, Boundary Value Analysis, Decision Table Testing, and State Transition Testing.

## 6.1 Equivalence Partitioning

Assumed valid renewable-energy generation range: 0–100 kW.

| Partition | Description | Representative Value | Expected Handling |
| --- | --- | --- | --- |
| EP-1 | Valid generation range | 50 kW | Accepted and displayed |
| EP-2 | Negative generation | -5 kW | Rejected as invalid |
| EP-3 | Above maximum | 150 kW | Rejected / abnormal alert |
| EP-4 | Zero generation | 0 kW | Accepted |
| EP-5 | Missing value | Null | Rejected |
| EP-6 | Non-numeric value | ABC | Rejected with format error |
| EP-7 | Valid decimal value | 25.5 kW | Accepted |

## 6.2 Boundary Value Analysis

| Test Point | Value | Expected Handling |
| --- | --- | --- |
| Minimum − 1 | -1 kW | Rejected |
| Minimum | 0 kW | Accepted |
| Minimum + 1 | 1 kW | Accepted |
| Nominal | 50 kW | Accepted |
| Maximum − 1 | 99 kW | Accepted |
| Maximum | 100 kW | Accepted |
| Maximum + 1 | 101 kW | Rejected |

Note: The 0–100 kW range is a testing assumption because the exact sensor-generation limits were not specified.

## 6.3 Decision Table Testing

| Rule | Generation vs Demand | Critical Shortage | Resulting Action |
| --- | --- | --- | --- |
| R1 | Generation > Demand | No | Use renewable + manage surplus |
| R2 | Generation = Demand | No | Use renewable energy |
| R3 | Generation < Demand | No | Use renewable + additional grid/backup |
| R4 | Generation < Demand | Yes | Additional grid/backup + critical alert |

## 6.4 State Transition Testing

Energy System State Transition Model

Normal → Warning → Critical → Recovery → Normal

Exception paths: Normal / Warning → Sensor Failure → Recovery; Normal / Warning → Network Failure → Reconnection → Normal

Normal: Generation is sufficient and the grid operates normally.

Warning: Generation begins to fall below expected demand.

Critical: Severe energy shortage or critical fault occurs.

Recovery: Generation or system connectivity is restored.

Sensor Failure: Sensor data is unavailable or invalid.

# 7. Detailed Test Case Design

As this is a test-case design exercise, no execution has been performed. Therefore, Actual Result = "Not Executed" and Pass/Fail Status = "Not Run". Test data is illustrative.

## 7.1 Authentication Test Cases

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-001 | FR-01 | Valid login | Open login → Enter credentials → Click Login | admin / Admin@123 | User is authenticated and redirected to dashboard. | Not Executed | Not Run |
| TC-002 | FR-01 | Invalid password | Enter valid username and wrong password → Login | admin / Wrong123 | Invalid-credentials message displayed. | Not Executed | Not Run |
| TC-003 | FR-01 | Invalid username | Enter unregistered username → Login | unknown / Admin@123 | Access denied with user-not-found message. | Not Executed | Not Run |
| TC-004 | FR-01 | Empty credentials | Leave fields blank → Click Login | Blank fields | Mandatory-field validation displayed. | Not Executed | Not Run |
| TC-005 | FR-01 | Multiple failed attempts | Enter incorrect password repeatedly | 5 failed attempts | Account is temporarily locked/warning displayed. | Not Executed | Not Run |

## 7.2 Sensor Management and Data Collection

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-006 | FR-03 | Register valid sensor | Open sensor management → Enter details → Save | Sensor ID: S-1001 | Sensor is successfully registered. | Not Executed | Not Run |
| TC-007 | FR-03 | Missing sensor ID | Leave sensor ID blank → Save | ID: blank | Mandatory-field error displayed. | Not Executed | Not Run |
| TC-008 | FR-04 | Valid sensor reading | Send valid sensor reading → Check dashboard | Solar: 50 kW | Reading is received and displayed. | Not Executed | Not Run |
| TC-009 | FR-04 | Zero generation | Send zero generation value | 0 kW | System accepts zero and displays 0 kW. | Not Executed | Not Run |
| TC-010 | FR-07 | Negative sensor value | Send negative reading | -5 kW | Reading is rejected as invalid. | Not Executed | Not Run |
| TC-011 | FR-07 | Non-numeric reading | Send text instead of numerical reading | ABC | Data-format error is displayed/logged. | Not Executed | Not Run |
| TC-012 | FR-07 | Missing sensor reading | Send null reading | Null | Missing-data error generated. | Not Executed | Not Run |
| TC-013 | FR-07 | Maximum valid reading | Send maximum supported value | 100 kW | Value is accepted. | Not Executed | Not Run |
| TC-014 | FR-07 | Above maximum reading | Send value above assumed limit | 101 kW | Value is rejected/abnormal alert generated. | Not Executed | Not Run |

## 7.3 Energy Monitoring

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-015 | FR-05 | Generation monitoring | Send solar reading → Open dashboard | 50 kW | Correct generation is displayed. | Not Executed | Not Run |
| TC-016 | FR-06 | Consumption monitoring | Send consumption reading → View dashboard | 40 kW | Correct consumption is displayed. | Not Executed | Not Run |
| TC-017 | FR-05 | Real-time generation update | Change sensor reading → Refresh dashboard | 50 → 65 kW | Dashboard displays latest reading. | Not Executed | Not Run |
| TC-018 | FR-06 | High consumption | Send high consumption reading | Demand: 95 kW | High-demand condition is identified. | Not Executed | Not Run |
| TC-019 | FR-08 | Store valid reading | Send valid reading → Check database | 60 kW | Reading is stored with timestamp and sensor ID. | Not Executed | Not Run |
| TC-020 | FR-08 | Duplicate reading | Send same reading twice | Same ID + timestamp | Duplicate is detected/handled according to rules. | Not Executed | Not Run |

## 7.4 Renewable Energy Prediction

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-021 | FR-09 | Valid prediction | Provide historical energy data → Run prediction | 30 days data | Future renewable generation prediction is generated. | Not Executed | Not Run |
| TC-022 | FR-09 | Insufficient data | Run prediction with insufficient historical data | 1 day data | System displays insufficient-data warning. | Not Executed | Not Run |
| TC-023 | FR-09 | Missing prediction input | Remove required input → Run model | Missing values | Appropriate validation/error message displayed. | Not Executed | Not Run |
| TC-024 | FR-09 | Prediction result display | Run prediction → Open dashboard | Predicted: 72 kW | Predicted value is correctly displayed. | Not Executed | Not Run |

## 7.5 Generation-Demand Analysis and Grid Optimization

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-025 | FR-10 | Generation greater than demand | Enter generation and demand | 80 kW / 50 kW | System identifies 30 kW surplus. | Not Executed | Not Run |
| TC-026 | FR-10 | Generation equals demand | Enter equal values | 50 / 50 kW | System identifies balanced condition. | Not Executed | Not Run |
| TC-027 | FR-10 | Generation lower than demand | Enter generation and demand | 40 / 70 kW | System identifies 30 kW shortage. | Not Executed | Not Run |
| TC-028 | FR-11 | Surplus optimization | Provide surplus renewable energy | 100 / 60 kW | System recommends storage/export/use of surplus. | Not Executed | Not Run |
| TC-029 | FR-11 | Shortage optimization | Provide insufficient renewable energy | 40 / 80 kW | System uses additional grid/backup energy. | Not Executed | Not Run |
| TC-030 | FR-11 | Critical shortage | Provide very low generation and high demand | 10 / 100 kW | Critical condition and appropriate action are triggered. | Not Executed | Not Run |
| TC-031 | FR-20 | Grid status monitoring | Open grid monitoring panel | Grid: Normal | Correct grid status is displayed. | Not Executed | Not Run |

## 7.6 Alerts and Abnormal Conditions

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-032 | FR-13 | Abnormal generation | Send sudden unrealistic reading | 10 → 500 kW | Abnormal reading is detected and logged. | Not Executed | Not Run |
| TC-033 | FR-14 | Warning alert | Create moderate energy shortage | 45 / 60 kW | Warning notification is generated. | Not Executed | Not Run |
| TC-034 | FR-14 | Critical alert | Create severe shortage | 10 / 100 kW | Critical alert is generated. | Not Executed | Not Run |
| TC-035 | FR-15 | Sensor failure | Disconnect sensor | Sensor offline | Sensor failure is detected and reported. | Not Executed | Not Run |
| TC-036 | FR-14 | Alert service failure | Disable notification service | Service unavailable | Monitoring continues and failure is logged. | Not Executed | Not Run |

## 7.7 Dashboard and Reporting

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-037 | FR-12 | Dashboard loading | Login → Open dashboard | Valid user | Dashboard loads correctly. | Not Executed | Not Run |
| TC-038 | FR-12 | Dashboard data visualization | Open charts | 24-hour energy data | Charts display correct trends. | Not Executed | Not Run |
| TC-039 | FR-17 | Energy history | Open historical energy view | Previous 7 days | Correct historical readings are displayed. | Not Executed | Not Run |
| TC-040 | FR-16 | Generate valid report | Select date range → Generate | 01-Aug-2026 to 07-Aug-2026 | Report is generated correctly. | Not Executed | Not Run |
| TC-041 | FR-16 | Empty report range | Select period without data | Future date range | Clear 'No data available' message displayed. | Not Executed | Not Run |
| TC-042 | FR-16 | Invalid date range | Select end date before start date | 07-Aug → 01-Aug | Date-range validation error displayed. | Not Executed | Not Run |

## 7.8 Security Test Cases

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-043 | NFR-02 | Unauthorized dashboard access | Open protected dashboard without login | Direct dashboard URL | Access denied and redirected to login. | Not Executed | Not Run |
| TC-044 | NFR-02 | Invalid session | Use expired session | Expired token | Request rejected with authentication error. | Not Executed | Not Run |
| TC-045 | NFR-02 | Role-based restriction | Login as normal user → Open admin panel | User role: Operator | Admin access denied. | Not Executed | Not Run |
| TC-046 | NFR-02 | Malicious input | Enter script/SQL-like input | <script>alert(1)</script> | Input sanitized/rejected; no execution. | Not Executed | Not Run |
| TC-047 | NFR-10 | Unauthorized data access | User A attempts User B's energy data | User B data request | Access denied. | Not Executed | Not Run |

## 7.9 Reliability and Exception Handling

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-048 | NFR-03 | Network interruption | Disconnect network during sensor update | Network unavailable | System displays connection error without corrupting data. | Not Executed | Not Run |
| TC-049 | NFR-03 | API failure | Simulate backend API error | HTTP 500 | Graceful error displayed. | Not Executed | Not Run |
| TC-050 | NFR-03 | Database failure | Make database unavailable during data storage | DB unreachable | Data is not partially stored and failure is logged. | Not Executed | Not Run |
| TC-051 | NFR-03 | Recovery after failure | Restore failed service | Service restored | System returns to normal operation. | Not Executed | Not Run |
| TC-052 | NFR-04 | Server restart | Restart server during monitoring | Server restart | System recovers and stored data remains available. | Not Executed | Not Run |

## 7.10 Performance, Scalability and Data Consistency

| TC ID | Req. ID | Test Scenario | Test Steps | Test Data | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-053 | NFR-01 | Dashboard response time | Open dashboard and measure response | Normal load | Dashboard responds within defined limit. | Not Executed | Not Run |
| TC-054 | NFR-01 | Real-time update performance | Send new sensor reading → Measure update | New reading | Dashboard updates within acceptable latency. | Not Executed | Not Run |
| TC-055 | NFR-06 | Multiple sensors | Send readings from multiple sensors simultaneously | 100 sensors | System processes readings without functional degradation. | Not Executed | Not Run |
| TC-056 | NFR-06 | High data load | Send large number of readings | 5000 readings | System processes data without significant failure. | Not Executed | Not Run |
| TC-057 | NFR-07 | Concurrent sensor updates | Multiple sensors send readings simultaneously | 20 concurrent sensors | Final stored values remain accurate and consistent. | Not Executed | Not Run |
| TC-058 | NFR-07 | Duplicate data handling | Resend same sensor request | Same ID + timestamp | Duplicate reading is not incorrectly stored twice. | Not Executed | Not Run |

# 8. Requirement Traceability Matrix

| Req. ID | Requirement | Scenario IDs | Test Case IDs | Coverage |
| --- | --- | --- | --- | --- |
| FR-01 | User Registration and Login | TS-01, TS-02 | TC-001–TC-005 | Fully Covered |
| FR-03 | Sensor Registration | TS-03 | TC-006–TC-007 | Fully Covered |
| FR-04 | Sensor Data Collection | TS-04 | TC-008–TC-009 | Fully Covered |
| FR-05 | Energy Generation Monitoring | TS-05 | TC-015, TC-017 | Fully Covered |
| FR-06 | Energy Consumption Monitoring | TS-06 | TC-016, TC-018 | Fully Covered |
| FR-07 | Sensor Data Validation | TS-07, TS-08 | TC-010–TC-014 | Fully Covered |
| FR-08 | Energy Data Storage | TS-09, TS-22 | TC-019–TC-020 | Fully Covered |
| FR-09 | Renewable Energy Prediction | TS-10 | TC-021–TC-024 | Fully Covered |
| FR-10 | Generation-Demand Comparison | TS-11 | TC-025–TC-027 | Fully Covered |
| FR-11 | Grid Optimization | TS-12 | TC-028–TC-030 | Fully Covered |
| FR-12 | Real-Time Dashboard | TS-16 | TC-037–TC-038 | Fully Covered |
| FR-13 | Abnormal Detection | TS-13 | TC-032 | Fully Covered |
| FR-14 | Alerts | TS-14 | TC-033–TC-036 | Fully Covered |
| FR-15 | Sensor Failure Detection | TS-15 | TC-035 | Fully Covered |
| FR-16 | Report Generation | TS-17 | TC-040–TC-042 | Fully Covered |
| FR-17 | Energy History | TS-16 | TC-039 | Fully Covered |
| FR-18 | Admin Monitoring | TS-18 | TC-031 | Fully Covered |
| FR-20 | Grid Status | TS-18 | TC-031 | Fully Covered |
| NFR-02 | Security | TS-19 | TC-043–TC-047 | Fully Covered |
| NFR-03 | Reliability | TS-20 | TC-048–TC-051 | Fully Covered |
| NFR-04 | Availability | TS-20 | TC-052 | Fully Covered |
| NFR-06 | Scalability | TS-21 | TC-055–TC-056 | Fully Covered |
| NFR-07 | Data Integrity | TS-22 | TC-019–TC-020, TC-057–TC-058 | Fully Covered |
| NFR-10 | Privacy | TS-19 | TC-047 | Fully Covered |

# 9. Test Coverage Analysis

| Metric | Value |
| --- | --- |
| Total Functional Requirements | 21 |
| Total Non-Functional Requirements | 10 |
| Total Test Scenarios | 22 |
| Total Test Cases | 58 |
| Requirements Fully Covered | 25 / 25* |
| Requirements Not Covered | 0* |
| Test Execution Status | Not Executed |

*Coverage is based on the requirements selected and mapped in this assignment. Requirements such as password reset and data export should receive additional test cases if those features are implemented in the final system.

Execution-based pass percentage is not calculated because this document represents test-case design rather than executed testing.

# 10. Security, Reliability and Quality Analysis

## 10.1 Security

Authentication before accessing monitoring and optimization functions.

Authorization and role-based access control.

Protection of energy-monitoring and account information.

Validation of sensor and user inputs.

Protection against malicious input.

Prevention of unauthorized access to energy history and reports.

## 10.2 Reliability

Graceful handling of network interruptions.

Detection and handling of sensor failures.

Recovery after backend API failures.

Recovery after database failures.

Preservation of previously stored energy data.

Logging of failure events for diagnosis.

## 10.3 Data Quality

Detection of missing sensor readings.

Rejection of malformed sensor data.

Detection of negative or out-of-range energy values.

Prevention of unintended duplicate readings.

Correct timestamp association with sensor readings.

Consistency between dashboard and stored energy data.

## 10.4 Usability

Clear presentation of energy generation and consumption.

Easy-to-understand charts and graphs.

Clear warning and critical alerts.

Simple presentation of grid status.

Understandable prediction results.

Easy access to historical energy information.

## 10.5 Sustainability

Testing the correctness of renewable-energy monitoring, prediction, and grid-optimization logic directly supports efficient energy management. Accurate energy measurements help reduce incorrect decisions, while supply-demand analysis supports effective use of renewable energy. Reliable alerts and failure handling also help operators respond quickly to energy shortages and abnormal conditions.

# 11. Conclusion and References

## 11.1 Conclusion

This document presents a comprehensive test-case design for the Smart Renewable Energy Monitoring and Grid Optimization Platform, covering functional and non-functional requirements, test-scenario identification, and detailed test cases across authentication, sensor monitoring, energy generation, consumption monitoring, prediction, supply-demand analysis, grid optimization, alerts, dashboards, reporting, security, reliability, scalability, and data consistency.

The application of Equivalence Partitioning and Boundary Value Analysis strengthens coverage of numerical energy inputs by systematically testing valid, invalid, and boundary conditions. Decision Table Testing captures the logical combinations involved in generation-demand analysis and grid optimization, while State Transition Testing verifies Normal, Warning, Critical, Recovery, and failure states. Together with the Requirement Traceability Matrix, these techniques provide broad requirement-driven coverage of functional correctness, data integrity, security, reliability, usability, and performance.

## 11.2 Assumptions

The renewable-energy generation range of 0–100 kW is a testing assumption because exact sensor limits were not specified.

Values used for energy demand, prediction, and grid optimization are illustrative test data.

Warning and critical energy-shortage thresholds are assumed for test-design purposes.

Actual machine-learning prediction accuracy is not claimed because model execution is outside this test-case design.

Actual execution results, screenshots, deployment outcomes, and production energy readings are not claimed.

All Actual Result fields are recorded as Not Executed and all Pass/Fail Status fields as Not Run.

## 11.3 References

[1] IEEE Std 829-2008, IEEE Standard for Software and System Test Documentation, IEEE, 2008.

[2] ISTQB, Certified Tester Foundation Level Syllabus, International Software Testing Qualifications Board, 2018.

[3] G. J. Myers, C. Sandler, and T. Badgett, The Art of Software Testing, 3rd ed., Wiley, 2011.

[4] P. Ammann and J. Offutt, Introduction to Software Testing, 2nd ed., Cambridge University Press, 2016.

[5] R. S. Pressman and B. R. Maxim, Software Engineering: A Practitioner's Approach, 9th ed., McGraw-Hill Education, 2019.